/*

Sketch generated by the Arduino IoT Cloud Thing "Untitled 2"

https://create.arduino.cc/cloud/things/044f8f3d-18db-48d6-91f8-d7747ce07cbe

Arduino IoT Cloud Variables description

The following variables are automatically generated and updated when changes are made to the Thing

bool buzzTest;

bool changeDoor;

bool checkDoor;

bool doorOpen;

bool isPowerOn;

bool powerToggle;

Variables which are marked as READ/WRITE in the Cloud Thing will also have functions

which are called when their values are changed from the Dashboard.

These functions are generated with the Thing and added at the end of this sketch.

*/

//Lawrence John Balanza

//CS 488 Final Project

#define trig 2

#define echo 3

#define buzzer 4

//bool changeDoor;

//bool doorOpen;

//bool powerToggle;

//bool testBuzzer;

//bool isPowerOn;

long duration;

int distance;

int doorStatus = 0; //door is closed

char choice;

int powerStatus = 0; //power starts at 0

#include "thingProperties.h"

void setup() {

pinMode(trig, OUTPUT); // Sets the trigPin as an Output

pinMode(echo, INPUT); // Sets the echoPin as an Input

pinMode(buzzer, OUTPUT);

isPowerOn = false;

// Initialize serial and wait for port to open:

Serial.begin(9600);

// This delay gives the chance to wait for a Serial Monitor without blocking if none is found

delay(1500);

// Defined in thingProperties.h

initProperties();

// Connect to Arduino IoT Cloud

ArduinoCloud.begin(ArduinoIoTPreferredConnection);

/*

The following function allows you to obtain more information

related to the state of network and IoT Cloud connection and errors

the higher number the more granular information you’ll get.

The default is 0 (only errors).

Maximum is 4

*/

setDebugMessageLevel(2);

ArduinoCloud.printDebugInfo();

}

void loop() {

ArduinoCloud.update();

// Your code here

if (choice == 'a'){//toggle power on and off

if(powerStatus == 0){//turn on

//if(powerToggle = true){

tone(buzzer, 100);

delay(100);

noTone(buzzer);

delay(100);

tone(buzzer, 500);

delay(100);

noTone(buzzer);

delay(100);

tone(buzzer, 1000);

delay(100);

noTone(buzzer);

powerStatus = 1;

Serial.println("Power on");

isPowerOn = true;

choice = 'x';

}

else{//turn off

tone(buzzer, 1000);

delay(100);

noTone(buzzer);

delay(100);

tone(buzzer, 500);

delay(100);

noTone(buzzer);

delay(100);

tone(buzzer, 100);

delay(100);

noTone(buzzer);

powerStatus = 0;

isPowerOn = false;

Serial.println("Power off");

choice = 'x';

}

}//end of choice 'a'

if (powerStatus == 1){//while power is turned on, make functions available

while (choice == 'x'){//default case, detect door change if power is on

ArduinoCloud.update();

//clears trig pin

digitalWrite(trig, LOW);

delayMicroseconds(2);

//set off trigger pin every 10 microseconds, set buzzer to low

digitalWrite(trig, HIGH);

delayMicroseconds(10);

digitalWrite(trig, LOW);

digitalWrite(buzzer,LOW);

//read echo pulse, in microseconds

duration = pulseIn(echo, HIGH);

//convert microsecond pulse to cm

distance= (duration*0.034/2);

if ((distance <= 5) && (doorStatus == 0)){ //If distance is within desired detection range

Serial.println("Door has been opened");

doorStatus = 1;

doorOpen = true;

digitalWrite(buzzer, HIGH);

delay(100);

digitalWrite(buzzer,LOW);

delay(100);

digitalWrite(buzzer, HIGH);

delay(100);

digitalWrite(buzzer,LOW);

delay(100);

digitalWrite(buzzer, HIGH);

delay(100);

digitalWrite(buzzer,LOW);

delay(1000);

}

else if ((distance <= 5) && (doorStatus == 1)){ //distance in the detection range but already open, closes door

Serial.println("Door has been closed");

doorStatus = 0;

doorOpen = false;

delay(1000);

}

else{ //if distance is not in detection range

digitalWrite(buzzer, LOW);

}

// Uncomment to check distance

//.print("Distance: ");

//Serial.println(distance);

}

if ((choice == 'b') && (checkDoor == true)){ //check door status

if(doorStatus == 0){//if door is closed

Serial.println("Door is currently closed");

digitalWrite(buzzer,HIGH);

delay(500);

digitalWrite(buzzer,LOW);

delay(500);

digitalWrite(buzzer,HIGH);

delay(500);

digitalWrite(buzzer,LOW);

choice = 'x';

}

else{//if door is open

Serial.println("Door is currently open");

digitalWrite(buzzer,HIGH);

delay(500);

digitalWrite(buzzer,LOW);

delay(500);

digitalWrite(buzzer,HIGH);

delay(500);

digitalWrite(buzzer,LOW);

delay(500);

digitalWrite(buzzer,HIGH);

delay(500);

digitalWrite(buzzer,LOW);

choice = 'x';

}

}//end of choice 'b;

if((choice == 'c') && (changeDoor == true)){//change door status

if(doorStatus == 0){//open the door

doorStatus = 1;

doorOpen = true;

Serial.println("Door is now open");

digitalWrite(buzzer,HIGH);

delay(1500);

digitalWrite(buzzer,LOW);

delay(500);

digitalWrite(buzzer,HIGH);

delay(500);

digitalWrite(buzzer,LOW);

delay(500);

digitalWrite(buzzer,HIGH);

delay(500);

digitalWrite(buzzer,LOW);

delay(500);

digitalWrite(buzzer,HIGH);

delay(500);

digitalWrite(buzzer,LOW);

choice = 'x';

}

else{//close the door

doorStatus = 0;

doorOpen = false;

Serial.println("Door is now closed");

digitalWrite(buzzer,HIGH);

delay(1500);

digitalWrite(buzzer,LOW);

delay(500);

digitalWrite(buzzer,HIGH);

delay(500);

digitalWrite(buzzer,LOW);

delay(500);

digitalWrite(buzzer,HIGH);

delay(500);

digitalWrite(buzzer,LOW);

choice = 'x';

}

}//end of choice 'c'

if((choice == 'd')&& (buzzTest == true)){//test buzzer

Serial.println("Testing buzzer");

ledBuzz = true;

tone(buzzer, 50);

delay(200);

noTone(buzzer);

delay(200);

tone(buzzer, 150);

delay(200);

noTone(buzzer);

delay(200);

tone(buzzer, 250);

delay(200);

noTone(buzzer);

delay(200);

tone(buzzer, 350);

delay(200);

noTone(buzzer);

delay(200);

tone(buzzer, 450);

delay(200);

noTone(buzzer);

delay(200);

tone(buzzer, 550);

delay(200);

noTone(buzzer);

delay(200);

tone(buzzer, 650);

delay(200);

noTone(buzzer);

delay(200);

tone(buzzer, 750);

delay(200);

noTone(buzzer);

delay(200);

tone(buzzer, 850);

delay(200);

noTone(buzzer);

delay(200);

tone(buzzer, 950);

delay(200);

noTone(buzzer);

delay(200);

tone(buzzer, 1000);

delay(200);

noTone(buzzer);

ledBuzz = false;

choice = 'x';

} //end of choice 'd'

}//end of power status'1'

}//end of loop

/*

Since ChangeDoor is READ_WRITE variable, onChangeDoorChange() is

executed every time a new value is received from IoT Cloud.

*/

void onChangeDoorChange()  {

// Add your code here to act upon ChangeDoor change

choice = 'c';

}

/*

Since BuzzTest is READ_WRITE variable, onBuzzTestChange() is

executed every time a new value is received from IoT Cloud.

*/

void onBuzzTestChange()  {

// Add your code here to act upon BuzzTest change

choice = 'd';

}

/*

Since PowerToggle is READ_WRITE variable, onPowerToggleChange() is

executed every time a new value is received from IoT Cloud.

*/

void onPowerToggleChange()  {

// Add your code here to act upon PowerToggle change

choice = 'a';

//powerToggle = false;

}

/*

Since CheckDoor is READ_WRITE variable, onCheckDoorChange() is

executed every time a new value is received from IoT Cloud.

*/

void onCheckDoorChange()  {

// Add your code here to act upon CheckDoor change

choice = 'b';

}